# CTF Writeup: Hardware Managing System

# Challenge Description

Recently one of our assembly line machines broke after working prolonged under too high temperature exposure. We use a very sophisticated Temperature-Workload-Management-System implemented directly in hardware that should have instantly responded by activating the emergency-off safety feature. After recent reports that the vendor might be involved in industrial sabotage, we suspect a deliberate hardware backdoor. We were able to get both an HDL copy of the system and a matching simulation program.

Files provided: CtfTask.v (Yosys-synthesized Verilog netlist), VCtfTask (Verilator simulation binary)

# Step 1: Understand the Simulation

First, run the simulation to understand how it works:

$ echo "001002003004005006007" | ./VCtfTask
[1] Input: '001', Power Level: '000'
[1] Before Temperature 000 => After Temperature 000
...
[8] Testing Emergency Shutdown:
[8] Input: '255', Power Level: '255'
[8] EMERGENCY SHUTDOWN ACTIVATED!
TEST SUCCESSFUL

The simulation accepts 7 rounds of 3-character input. After the 7 user rounds, it forces a temperature of ≥100 and sends 255 as the workload — the emergency shutdown should activate. Our goal: find an input that causes TEST FAILED, EMERGENCY SHUTDOWN WAS NOT ACTIVED!

# Step 2: Reverse the Binary

The Verilog netlist is a gate-level description — understanding the timing requires examining the compiled simulation. Disassembling VCtfTask reveals key behaviour in sendByte and main:

// sendByte sends 2 clock ticks per byte:
dut->io_input = value & 0x7f;
dut->io_inputDone = 1;
tick(dut);   // rising edge
dut->io_inputDone = 0;
tick(dut);   // falling edge

// For each of 7 rounds, main() sends 4 bytes:
sendByte(dut, buffer[3*i+0]);  // digit 1 (ASCII)
sendByte(dut, buffer[3*i+1]);  // digit 2 (ASCII)
sendByte(dut, buffer[3*i+2]);  // digit 3 (ASCII)
sendByte(dut, 0x0a);           // HARDCODED 0x0A = 10
emulateTemperature(dut, i);

Critical: each round sends 4 bytes, not 3. The 4th byte is always the hardcoded value 10 (0x0A). This aligns perfectly with the state machine — 4 bytes × 4 states = one complete cycle per round.

# Step 3: Find the Backdoor LUTs

The Verilog file is a Yosys-synthesized netlist using parameterized LUT4/LUT5/LUT6 primitives. Each LUT module is named by a hash of its INIT constant. Searching for anomalous constant-output modules reveals two suspects:

// Always outputs 0 — regardless of inputs:
module $paramod$ceb62c6ee...\LUT6 (O, I0..I5);
  assign O = 1'h0;
endmodule

// Always outputs 1 — regardless of inputs:
module $paramod$e674067559...\LUT6 (O, I0..I5);
  assign O = 1'h1;
endmodule

A LUT that ignores all its inputs is a hardware backdoor signature. These replace normal comparison logic with constant outputs, silently corrupting the circuit's behaviour.

# Step 4: Trace the Emergency Shutdown Path

The top module signature is:

module CtfTask(io_input[6:0], io_inputDone, io_temp[7:0], io_power[7:0], clk, reset)

Tracing io_power backwards through the netlist reveals:

// io_power is gated by _270_:
// _270_ = NOR(_271_, _211d3dff[7])
// When _211d3dff[7] = 1, _270_ = 0, which DISABLES the shutdown

_211d3dff is an 8-bit register chain. Its bit 7 is the backdoor enable: once set, it masks the temperature comparison and prevents emergency shutdown.

# Step 5: Trace the 8-bit Activation Chain

_211d3dff[7] is driven by a LUT6 that evaluates to _226_ & _261_:

• _261_ = rising edge of io_inputDone at state=11, with io_input=10 — fires automatically every round at the hardcoded 0x0A byte

• _226_ = AND of six sub-conditions requiring specific register values AND _211d3dff[3]=1

The _226_ check requires _211d3dff[3] to already be set, creating a chain dependency. Tracing fully through the netlist, each _211d3dff[n] bit requires the previous prerequisite bit to be set, forming a sequential 7-step unlock sequence:

bit[4] → bit[2] → bit[6] → bit[1] → bit[5] → bit[3] → bit[7] (TRIGGER)

Each bit is set by specific register values at the 0x0A byte trigger moment, and holds via feedback through the always-1 LUT (the second backdoor LUT).

# Step 6: Decode Each Stage's Required Input

The three registers loaded per round are:

• reg_C: captured at the rising edge of byte 1 (state=00)

• reg_B: captured at the rising edge of byte 2 (state=01)

• reg_A: captured at the falling edge of byte 3 (state=10→11 transition)

Below is the full gate-level derivation for Round 1 (bit[4]) as a worked example. The same procedure applies to every subsequent round.

## Worked example — Round 1 / bit[4]

FDCE _736_ (CtfTask.v:4610) holds _211d3dff[4], D input = _193_:

LUT3 _464_ (INIT=8'11111000)   I0=_213_  I1=_264_  I2=_266_   (CtfTask.v:2909)

INIT binary 11111000: bits 3-7 are 1, so output = I2 | (I0 & I1) = _266_ | (_213_ & _264_). _266_ is a MUXF8 with select=_211d3dff[4]: outputs 0 when bit is unset, 1 when already set (self-latch). Initial arming condition: _213_ & _264_.

_264_ — five simultaneous register comparisons (CtfTask.v:2886):

LUT5 _461_ (INIT=32'10000000000000000000000000000000)   I0..I4 = _234_,_235_,_236_,_237_,_238_
Only bit[31]=1 → all five inputs must be simultaneously 1.

| Signal | Gate / INIT | Firing condition | Required register bits |
| --- | --- | --- | --- |
| _234_ | LUT4 :2237 INIT=16'0000000100000000 | bit[8]=1 {I3,I2,I1,I0}=1000 | reg_A[1]=1 reg_A[3]=reg_A[2]=reg_A[0]=0 |
| _235_ | LUT4 :2244 (same INIT) | bit[8]=1 {I3,I2,I1,I0}=1000 | reg_B[2]=1 reg_B[3]=reg_B[1]=reg_B[0]=0 |
| _236_ | LUT3 :2251 INIT=8'00010000 | bit[4]=1 {I2,I1,I0}=100 | reg_C[6]=1 reg_C[4]=reg_C[5]=0 |
| _237_ | LUT6 :2257 body: ~I0&~I1&~I2&I3&I4&I5 | direct | reg_B[4]=1, reg_B[6]=1, reg_A[6]=1 reg_A[4]=reg_A[5]=reg_B[5]=0 |
| _238_ | LUT5 :2266 INIT=32'd16777216 (bit[24]) | {I4,I3,I2,I1,I0}=11000 | _211d3dff[0]=1 (FDPE preset, always 1) reg_C[3]=1; reg_C[2]=reg_C[1]=reg_C[0]=0 |

Assembling all bit constraints:

reg_A [6:0] = 1 0 0 0 0 1 0 = 0b1000010 = 66 = 'B'  (from _237_ and _234_)
reg_B [6:0] = 1 0 1 0 1 0 0 = 0b1010100 = 84 = 'T'  (from _237_ and _235_)
reg_C [6:0] = 1 0 0 1 0 0 0 = 0b1001000 = 72 = 'H'  (from _236_ and _238_)

_213_ (timing strobe): LUT2 _307_ (INIT=4'0100) = _221_ & ~_214_. Fires when state=11 AND io_inputDone is rising — exactly when the hardcoded 0x0A byte is strobed. By that point reg_C, reg_B, and reg_A have already been captured from the three preceding bytes.

Chain dependency: bit[4] fires first because its only prerequisite is _211d3dff[0] (FDPE preset, always 1). For each later stage, the comparison LUT tree contains _211d3dff[k] for the previous stage as a required input — visible at CtfTask.v:2407 where _211d3dff[4] is wired into bit[2]'s arming logic. Order: bit[4]→bit[2]→bit[6]→bit[1]→bit[5]→bit[3]→bit[7].

Applying the same decode procedure to the remaining six rounds yields the full stage table:

| Round | Sets bit | reg_C (byte 1) | reg_B (byte 2) | reg_A (byte 3) | Input |
| --- | --- | --- | --- | --- | --- |
| 1 | [4] | H (72) | T (84) | B (66) | HTB |
| 2 | [2] | { (123) | B (66) | C (67) | {BC |
| 3 | [6] | K (75) | D (68) | 0 (48) | KD0 |
| 4 | [1] | 0 (48) | 0 (48) | R (82) | 00R |
| 5 | [5] | D (68) | _ (95) | H (72) | D_H |
| 6 | [3] | R (82) | D (68) | W (87) | RDW |
| 7 | [7] | R (82) | 3 (51) | } (125) | R3} |

# Step 7: Trigger the Backdoor

The unlocking input is the flag itself: HTB{BCKD000RD_HRDWR3} — 21 characters, seven rounds of three ASCII characters each. Decoding the LUT INIT constants and MUX trees for each prerequisite bit yields the exact required values:

| Round | Sets bit | Bytes sent (ASCII) | Input |
| --- | --- | --- | --- |
| 1 | [4] | H (72), T (84), B (66) | HTB |
| 2 | [2] | { (123), B (66), C (67) | {BC |
| 3 | [6] | K (75), D (68), 0 (48) | KD0 |
| 4 | [1] | 0 (48), 0 (48), R (82) | 00R |
| 5 | [5] | D (68), _ (95), H (72) | D_H |
| 6 | [3] | R (82), D (68), W (87) | RDW |
| 7 | [7] | R (82), 3 (51), } (125) | R3} |

Each round also sends the hardcoded 0x0A byte as byte 4, which strobes io_inputDone at state=11 and advances the state counter. After all seven rounds, _211d3dff[7] is latched high and the shutdown is disabled.

$ printf "HTB{BCKD000RD_HRDWR3}" | ./VCtfTask
...
[8] EMERGENCY SHUTDOWN MISSED!
TEST FAILED, EMERGENCY SHUTDOWN WAS NOT ACTIVED!
INPUT 'HTB{BCKD000RD_HRDWR3}' NEEDS FURTHER ANALYSIS

# Flag

HTB{BCKD000RD_HRDWR3}

# Summary

The backdoor consists of two injected constant-output LUT6 modules in a Yosys-synthesized netlist:

1. An always-0 LUT that nullifies register comparison outputs, preventing normal detection of the unlock sequence.

2. An always-1 LUT that provides a persistent feedback path, making bits of the activation chain latch once set.

Together they implement a hidden 8-bit shift register (_211d3dff) that arms when a specific 7-round ASCII sequence is provided. On the 8th round (the emergency temperature test), the armed backdoor suppresses the shutdown signal — a realistic hardware supply-chain attack that would be invisible to functional testing and only detectable through netlist inspection.